Based on our complete setup, here is the master wiring guide for your ESP32, A7670C 4G Module, Nextion HMI, Sensors, and Relays.

1. Power Supply (Critical)

- External 5V/3A Power Source: Connect to the VCC of the A7670C, the Relay Board, and the Nextion Display.

- Common Ground: Connect the GND of the External Power Supply, ESP32, A7670C, Nextion, and all sensors together.

2. Communication & Display

| Component | ESP32 Pin | Logic | Notes |
| --- | --- | --- | --- |
| A7670C TX | GPIO 16 (RX2) | 3.3V/1.8V | Receives data from 4G module |
| A7670C RX | GPIO 17 (TX2) | 3.3V/1.8V | Sends commands to 4G module |
| Nextion TX | GPIO 32 (RX1) | 3.3V | Receives data from HMI |
| Nextion RX | GPIO 33 (TX1) | 3.3V | Sends data to HMI |

3. Sensors

| Sensor | ESP32 Pin | Connection | Notes |
| --- | --- | --- | --- |
| DS18B20 (Data) | GPIO 4 | OneWire | Must have 4.7kΩ resistorto 3.3V |
| SHT45 (SDA) | GPIO 21 | I2C | Data line |
| SHT45 (SCL) | GPIO 22 | I2C | Clock line |
| Float Switch | GPIO 13 | Digital | Black to GND, Blue to Pin 13 |

4. 7-Channel Relays (Normally Closed - NC)

- Wiring: Connect the device "Live" wire to COM and NC.

- Operation: LOW = Device ON (Default), HIGH = Device OFF (Commanded).

| Relay | ESP32 Pin | Function |
| --- | --- | --- |
| Relay 0 | GPIO 2 | Work Light |
| Relay 1 | GPIO 12 | Primary Heater |
| Relay 2 | GPIO 14 | Secondary Heater (Boost) |
| Relay 3 | GPIO 15 | Tilt UP |
| Relay 4 | GPIO 25 | Tilt DOWN |
| Relay 5 | GPIO 26 | Primary Fan |
| Relay 6 | GPIO 27 | Secondary Fan (High Speed) |

5. Nextion Components Checklist

Ensure your .HMI file uses these exact object names for the code to sync:

- x0: Average Temperature (vvs1=1)

- x2: Humidity (vvs1=1)

- n0: Temperature Threshold (Set via T%d\n)

- n1: Humidity Threshold (Set via H%d\n)

- b9: Tank Status Text ("EMPTY" / "FILLED")

- bt0: Work Light Status Toggle

Would you like a schematic diagram description for how to wire the Secondary Fan for its two speeds?